ON ROAD DRIVING TEST

Turns

As You Approach the Turn:

- Check traffic in all directions.

- Turn signal on.

- Decelerate/slow down, change gears as needed to keep vehicle under power.

- Do not coast with vehicle in neutral or clutch depressed for more than one vehicle length.

- Be in the correct lane for the turn.

If Stopping Before Turn:

- Necessary, was the stop necessary? Example: stopping on a green light.

- Smooth stop, no skidding, and smooth easy braking.

- Stop line, stop behind the stop line, crosswalk, or stop sign.

- Gap, (safe gap) is where you can see the rear tires on the ground of the vehicle ahead of you.

- Full Stop, keeps foot on the brake, do not ride the clutch or have vehicle in neutral.

- Roll back or forward, do not allow the vehicle to roll in any direction.

- Wheels straight, keep the front wheels aimed straight ahead.

Turning:

- Traffic check in all directions before executing the turn.

- Both hands on the wheel, no palming or one-handed steering

- Gears, no shifting unless necessary or when the vehicles steering wheels are straight.

- Speed, not too fast, as to where the vehicle pushes you around the turn with too much lateral motion or too slow, not to impede other traffic.

- Yield to other traffic, pedestrians etc.

- Mirrors, monitor trailer/vehicle completely around turn until straight.

Completion of Turn:

- Traffic check, check mirrors after the turn.

- Correct lane, you must complete the turn left-into-left and right-into-right lanes.

- Finish right lane, after a left-to-left turn you must work your way into the right lane.

- Signal off

- Accelerate and get up to speed of traffic.

Intersections:

Approach:

- Traffic check, check traffic thoroughly on all directions.

- Decelerate gently

- Coast, take foot off of accelerator and cover the brake on a stale green light or when there is a potential hazard on a non-controlled intersection.

- Correct lane

Stopping at an Intersection:

- Necessary, was the stop necessary, timing of traffic lights.

- Smooth, was the stop a smooth stop no skidding.

- Gap

- Stop Line

- Full stop, foot on the brake, not riding clutch or in neutral.

- Rolling, not rolling in either direction.

Driving Through an Intersection

- Traffic check

- Both hands on the wheel

- Gears, no shifting unless necessary

- Yield to any pedestrians and traffic in intersection.

- Lane, do not change lanes while proceeding through intersection.

- Outage, make sure your vehicle can clear the whole intersection.

Completion, Once Through the Intersection:

- Traffic check, continue checking traffic and mirrors.

- Correct lane

- Accelerate smoothly and change gears as necessary.

Lane Changes

- Traffic checks before, during and after the lane change

- Signal on

- Spacing, not tailgating when and after a lane change

- Smooth change, a controlled drift

- Cancel signal

Urban and Rural Highways

- Regular traffic checks

- Checks cross traffic

- Proper lane, right most lane.

- Keeps vehicle centered in lane

- Speed, keep up of with the flow of traffic but do not exceed speed limit.

- Following distance, one second for every 10 foot of vehicle length.

Expressway

Merge On:

- Traffic check

- Signal

- Spacing, do not tailgate while merging on expressway

- Merge smoothly into the proper lane of traffic.

- No stopping on ramp, adjust speed to blend with traffic.

Expressway Driving:

- Traffic checks, continually checks traffic in all directions.

- Lane, maintain proper lane positioning, right or center lane.

- Speed, maintain speed limit.

- Following distance, 1 second for every 10 foot of vehicle length, 1 more second for speeds over 40 mph and 1 more second for adverse weather.

Exit:

- Traffic checks

- Signal

- Spacing, adequate spacing between you and other vehicles.

- Merge smoothly onto exit ramp.

- Decelerate in exit lane if one is provided. Do not go under minimum speed of 40 mph while still on interstate.

- Ramp speed, Decelerate on ramp do not exceed ramp speed.

Curves

- Traffic check, when approaching a curve check traffic thoroughly.

- Speed, before entering the curve, reduce speed so further braking or shifting is not required.

- Keep vehicle in lane, do not cross over lane lines.

- Traffic checks, continue checking traffic in all directions.

Road Side Stop

Approach

- Traffic check

- Signal on

- Correct lane

- Decelerate smoothly and brake evenly.

- Coast, bring your vehicle to a full stop without coasting.

Stop

- Parallel to the curb or shoulder of the road, safely out of the flow of traffic.

- Not blocking driveways, fire hydrants, intersections, signs etc.

- Do not allow vehicle to roll.

- Cancel turn signal and activate 4-way flashers.

- Apply the parking brake

- Move shifter into neutral or park

- Remove your feet from the brake and clutch.

Resume

- Traffic check, check traffic and mirrors thoroughly in all directions.

- Turn off 4-way flashers

- Activate turn signal

- Do not stall engine

- Do not roll forwards or backwards while waiting to pull out.

- Release parking brake.

- Check traffic from all directions, especially to the rear.

- Pull out when it is safe to do so.

- Accelerate smoothly and into the proper lane.

- Cancel turn signal.

Signs:

Check for signs, especially signs that apply to a commercial vehicle.

General Driving Behaviors:

- Used clutch properly, doubled-clutched, did not ride the clutch, synchronized clutch and shifter.

- Used gears properly, did not over rev or lug engine, did not clash gears.

- Used brakes properly, smooth braking no riding or pumping.

- Proper steering, both hands on wheel, no over/under control of vehicle.

- Proper lane usage, used correct lanes, not over lane lines stop lines etc.

- Used regular traffic checks

- Proper use of turn signals.

Railroad Crossing Placarded for Hazardous Materials or Passengers

Approach:

- Look and listen for the presence of trains

- Check traffic in all directions

- Activate 4-way flashers

- Decelerate, brake smoothly, and shift gears as necessary

Stopping:

- Stop no closer than 15 feet or any further than 50 feet to the nearest rail.

- Come to a full stop

- Open the window to listen for an approaching train

- Look and listen for the presence of trains

- Check traffic in all directions

Crossing:

- Do not stop, change gears, or change lanes with any part of the vehicle is proceeding across the tracks

- Look and listen for the presence of trains

- Check traffic in all directions

- “Outage” Must be able to get the entire vehicle across the tracks without stopping

Completion:

- Four-way flashers should be deactivated after the vehicle crosses the tracks.

- Check traffic in all directions.

- Correct lane should be the right most curb lane

- Driver accelerates and gets vehicle across the tracks.

ROADSIDE STOP PROCEDURES

- Activate directional signals

- Traffic Check

- Park parallel to curb in a safe location, not blocking traffic

- Activate 4 way flashers

- Set Air parking brake and place vehicle into natural or park

- Emergency pull-overs require reflective triangle to be set out after 10 minutes

- Returning to roadway

- Cancel 4 way flashers

- Activate directional signal

- Shift into proper gear

- Check mirrors before proceeding back on the roadway